Data set IIIb: Non-stationary 2-class BCI dataGroup information:Center of NeuroinformaticsSchool of Life Science and TechnologyUniversity of Electronic Science and Technology of China (UESTC)Chengdu, 610054, P. R. ChinaSupervisor: Dezhong yao  E-mail: dyao@uestc.edu.cnTeam Member: Yu Yin ,Xiang LiaoDescription of our analysis method Processing Procedure: Select subject-specified scales of the complex Morlet wavelet by a statistical method;Transform the EEG data s(t) with the complex Morlet wavelet on the selected scale, and calculate the squared wavelet coefficient as the element of the following feature vector.x(s,τ)=s(t)∗ψs,τ2 , is the mother wavelet.The wavelet transform was conducted on a time window, and the result was assumed for the point of the front boundary of the window.(3) Construct the feature vector of electrodes C3 and C4 for every sampling point, and normalize each feature vector to an interval of [0 1];(4) Classify each sample point by an adaptive Support Vector Machines (SVM), with a result denoted by St(x) belongs to [-1 1]. (5) Accumulate all the previous results (before time point t), a normalized weighted summation, as the classification result of the current time point:St0(x)=t=1t0wtSt(x)t=1t0wtwt is the weight factor determined by a parameter of SVM.